BAS ON-BOARD AUTONOMOUS HAR SYSTEM

System Design: Orientation-Agnostic HAR + Digital Twin + Multi-Agent Validation

Instructor Brief — Proposed Architecture

# 1. Problem & Objective

The proposed system is an offline, on-board AI assistant for microgravity experiments on the Bharatiya Antariksh Station (BAS). It continuously analyzes fixed-camera video, understands astronaut–object interactions, identifies the current experimental step, and validates the procedure without depending on continuous ground support.

Recognize astronaut activities and hand–object interactions from local video.

Track the astronaut in an orientation-agnostic 3D payload-rack coordinate frame.

Maintain a live Digital Twin representing the astronaut, objects, rack and experiment state.

Validate the prescribed sequence using a deterministic Finite State Machine (FSM).

Use multiple specialized agents for perception, twin synchronization, reasoning, guidance and logging.

Provide local voice guidance/alerts and lightweight structured telemetry.

# 2. Design Principles

| Principle | Design Choice | Why it matters |
| --- | --- | --- |
| Edge-first | Local inference on Jetson Orin NX / spaceflight compute | Reduces dependence on bandwidth and ground availability. |
| Orientation-agnostic | 3D HMR + fixed payload-rack frame | Astronauts can work in arbitrary/inverted orientations. |
| Safety by design | FSM remains deterministic | LLM/agents cannot directly decide procedural validity. |
| State-aware | Digital Twin | Maintains a consistent view of the whole experiment. |
| Modular AI | Specialized agents | Each agent has a clear responsibility and can be independently tested. |

The thesis describes fixed-camera local processing, orientation-agnostic 3D HMR, HOI-based validation, deterministic FSM logic, offline TTS, and edge deployment. The Digital Twin + explicit multi-agent orchestration are presented here as the integrated system-design layer.

3. MAIN SYSTEM DESIGN

FIXED PAYLOAD CAMERA Local video stream

↓

1. EDGE VIDEO PIPELINE Frame capture • preprocessing • temporal buffering

↓

| OBJECT PERCEPTION YOLOv8-nano • Object class • Bounding box • Color | 3D HUMAN PERCEPTION Orientation-agnostic HMR • 3D joints • Body/hand pose • Rack-relative coordinates | TEMPORAL PERCEPTION AdaSpot + lightweight HAR • Active RoI • Action sequence • Event timing |
| --- | --- | --- |

↓

2. PERCEPTION FUSION + HAND–OBJECT INTERACTION Fuse object location, 3D hand position, distance/contact and temporal evidence

↓

3. DIGITAL TWIN ENGINE Live virtual payload rack + astronaut + objects + current experiment state

↓

| 4. MULTI-AGENT ORCHESTRATOR Coordinates specialized agents using the Digital Twin as shared state |  |  |  |
| --- | --- | --- | --- |
| Perception Agent Consumes vision outputs and updates observations. | Twin Agent Maintains object, pose and experiment state. | Reasoning Agent Explains context and supports next-step guidance. | Logging/Guidance Agent Creates telemetry and voice/text feedback. |

↓

5. DETERMINISTIC FSM VALIDATOR Expected procedure state ↔ observed Digital Twin state ✓ Valid • ⚠ Out-of-order • ⚠ Skipped

↓

6. OUTPUTS Next-step guidance • Offline TTS alert • GUI • Timestamped JSONL • Local video / stream

Safety boundary: the multi-agent/LLM layer can interpret, summarize and recommend, but the FSM is the authoritative component for sequence compliance.

4. HOW THE SYSTEM WORKS — SIMPLE EXAMPLE

Example experiment: Open container → Extract Red Box → Extract Yellow Box. The same architecture can be adapted to other BAS payload procedures.

| Step | What the AI observes | Digital Twin state | Validation / action |
| --- | --- | --- | --- |
| 1. Open container | Lid movement + object context | Container = OPEN | FSM → State 1 ✓ |
| 2. Extract red | Hand approaches red box; box exits container bounds | Red = EXTRACTED | FSM → State 2 ✓ |
| 3. Extract yellow | Hand approaches yellow box; box exits bounds | Yellow = EXTRACTED | FSM → COMPLETE ✓ |
| Wrong order | Yellow detected while red is required | Yellow = TOUCHED/HELD | FSM → ERROR + voice alert |

# 5. Core AI Modules

Object Detection: Customized YOLOv8-nano, INT8/TensorRT for local inference; detects experiment-specific objects and supports color-aware identification.

3D Human Reconstruction: Orientation-agnostic HMR with a fixed payload-rack coordinate frame; PhysAstro-Pose concepts such as COC, S-PLR and URCF improve robustness to floating/inverted postures.

Computational Saliency: AdaSpot-style low-resolution context + active high-resolution RoI processing to reduce unnecessary computation.

Temporal HAR: Lightweight temporal modeling such as μBi-ConvLSTM/LITEWAY can be evaluated for edge deployment; the final model should be selected from measured latency/accuracy.

Hand–Object Interaction: Combines 3D hand joints, object boxes and spatial/temporal thresholds to infer grasp, extraction and manipulation events.

FSM Validator: Encodes allowed experiment transitions and requires persistent evidence before committing a state change, reducing false alerts.

# 6. Edge & Software Deployment

Development prototype: PC + Python + Blender/Three.js Digital Twin + webcam/video input.

Edge target: NVIDIA Jetson Orin NX for vision/HMR workloads; the thesis specifies a 10–25 W operating envelope for the module.

Safety-critical validation/logging should remain isolated from probabilistic AI services; the thesis proposes partitioning perception and deterministic functions.

GUI: live video, 3D twin/skeleton, current step, alerts and diagnostics.

Telemetry: log only structured events, timestamps, state and outcomes instead of relying on continuous raw-video transmission.